MACRO-PROCESS: PACKAGING

Subprocess B4: Determination of the Moisture Percentage

| OBJECTIVE | This procedure describes the tasks to be performed to determine the moisture content in the case of ex situ seed conservation. |  |
| --- | --- | --- |
| ACCOUNTABLE | Curator |  |
| RELATIONS | SUPPLIERS | Curator |
|  | CLIENTS | Curator |
| RECORDS | INPUT | Clean, sanitized sample with viability control performed |
|  | OUTPUT | Percentage of moisture, Remains of seeds used for the test, Completed sample form. |
| ENVIRONMENTAL FACTORS | FACILITIES AND WORKING ENVIRONMENT | Genebank facilities. Workshops and Laboratory. |
|  | ENVIRONMENTAL IMPACT | Electricity consumption, paper consumption, water consumption. |
| ASSOCIATED DOCUMENTATION | - Instruction Manual for PRECISA electronic moisture meter, model XM~60 |  |
| INDICATORS | INDICATOR |  |
|  | ACCOUNTABLE |  |
|  | FREQUENCY |  |
|  | PRESENTATION |  |
|  | RELATIONSHIP WITH OBJECTIVES |  |

INDICATIONS

(1)	The process of determining the moisture content of a seed sample carried out on an electronic moisture analyzer. This analyzer consists mainly of a heat chamber and a scale. This is a destructive test, so the number of seeds we have has to be high in order to be able to carry out this measurement. Grind as quickly as possible (with the help of an electric mill) between 10-12 grains of seeds, to prevent them from absorbing moisture. Once in the device, the sample will be heated at about 130ºC, and the time will vary according to the type of seed. Finally, depending on the difference in weight, the device gives us a direct reading of the percentage of moisture in the sample.

(2)	We count as optimal conditions, both the date and the availability of suitable land, etc.

START

Is there enough sample to perform analysis?

Moisture Analysis (1)

Is planting possible? (2)

Dissection

Multiplication

END

Yes

Clean, sanitized sample with ctrl viability

% moisture

Completed form

Disposal of samples used for the test

Yes

No

No